Jan 23, 2025

BLE Mesh based IR AC Controller

Software Design Document

Revision History

| Version | Release Date | Prepared by | Remarks |
| --- | --- | --- | --- |
| 0.2 | 01.04.2024 | Kulasekaran | - Draft Version |
| 0.3 | 10.04.2024 | Kulasekaran | - Removed the Heartbeat related content as temperature data has been decided to be taken as heartbeat - Added Teaching mode packet and ACK for Gwy and Node |
| 0.4 | 24.04.2024 | Kulasekaran | - Changed the Temperature upper and lower limit keys |
| 0.5 | 24.06.2024 | Kulasekaran | - Added Solid Purple indication for when sending out IR command in both Gwy and Node - Added packet information regarding Teaching modes - Added Boot up LED pattern information for both Gwy and Node in the LED Table - Modified Button press logics - Added Debug Info packets for Gwy and Node - Changed Error code values - Minor changes with more clear descriptions to certain sections across the whole document - Added Detailed Error Code information - LED indication change for Teaching mode and AC Remote configuration mode. |
| 0.6 | 30.07.2024 | Kulasekaran | - Added ResetDevice and Logging to the Error code - Added Unsupported Remote LED indication - Removed NODE_ALREADY_PROV error code - Added AC_REMOTE_UNSUPPORTED error code - The default Heartbeat interval increased from 10s to 300s. - Added JSON_PACKET_INVALID error code |
| 0.7 | 01.08.2024 | Kulasekaran | - ACK responses simplified. Unnecessary fields removed. - Change to Button press logics |
| 0.8 | - | - | - - |
| 0.9 | 03.12.2024 | Kulasekaran | - Node provisioning flow modified - Button press functionalities modified - Error codes modified - JSON Packet modified - Teaching mode modified - LED indications modified |
| 1.0 | 24.01.2025 | Kulasekaran | - Added Invalid format error codes |

# Introduction

This Software Design Document outlines the technical architecture, design components, and implementation strategy for the project “IR BLE Mesh AC Controller”. It serves as a comprehensive guide to ensure the system aligns with the client's functional requirements, scalability goals, and performance expectations. The document details key design decisions, system interactions, and data flow, providing clarity for all stakeholders. Our approach prioritizes modularity, reliability, and adaptability to accommodate future enhancements. This document will act as a reference throughout development, ensuring consistency and traceability from design to deployment. Feedback is encouraged to confirm alignment with the client’s objectives and to address any concerns proactively.

## Purpose of Document

The purpose of this Software Design Document (SDD) is to provide a detailed blueprint for the development of the project “IR BLE Mesh AC Controller”. It ensures a shared understanding of the system’s architecture, design principles, and implementation strategy among all stakeholders, including the client, development team, and QA team. The goal is to deliver a solution that aligns with the client’s objectives while adhering to industry best practices.

## Scope of the Project

## Acronyms, Abbreviations and Definitions

| Acronym | Full Form |
| --- | --- |
| ACK | Acknowledgement |
| UART | Universal Asynchronous Receiver Transmitter |

| Abbreviation | Full Form |
| --- | --- |
| BLE | Bluetooth Low Energy |
| GWY | Gateway |
| IR | Infrared |
| JSON | Java Script Object Notation |
| LTE | Long Term Evolution |
| MQTT | Message Queuing Telemetry Transport |

| Term | Description |
| --- | --- |
| Registration | Adding a Gateway device |
| Unregistration | Removing a Gateway device |
| Provisioning | Adding a Node device into BLE Mesh Network |
| Unprovisioning | Removing a Node device from BLE Mesh Network |
| Configuration | Process of telling the device, which AC remote to act as |

## References

## Overview of the Document structure

# System Overview

The project “IR BLE Mesh AC Controller” is designed to enable seamless control and monitoring of air conditioners. The product comes in two variants: the Gateway and the Node, which work together to provide a comprehensive solution for smart AC management.

## System Architecture

- Gateway (Master):

  - Equipped with LTE capability for internet connectivity.

  - Connects to an MQTT broker, enabling communication through MQTT topics.

  - Acts as the central controller of the BLE mesh network.

  - Updates the system status to the cloud and processes user commands from the mobile app via MQTT.

- Node (Slave):

  - Functions as a BLE mesh device communicating with the Gateway.

  - Executes commands relayed by the Gateway, such as adjusting AC settings or retrieving sensor data.

- Mobile App:

  - Connects to the same MQTT broker as the Gateway.

  - Allows users to send commands and receive real-time status updates from the Gateway via MQTT topics.

## Key Features

- BLE Mesh Communication:

  - Establishes reliable and efficient communication between Gateway and multiple Nodes within the network.

- MQTT Integration:

  - Facilitates seamless interaction between the Gateway and the mobile app, enabling remote control and monitoring.

## Use Cases

- Remote Control via Mobile App:

  - Users can adjust AC settings or query system status through MQTT-based communication with the Gateway.

- System Notifications:

  - Fault detection and system status updates are pushed to the mobile app through MQTT topics for real-time alerts.

This system integrates BLE mesh communication for local functionality with MQTT-based cloud connectivity, providing a scalable and efficient solution for modern AC management.

## Architecture Design

The Architecture Design section provides a comprehensive overview of the system’s high-level structure, detailing the key components, their responsibilities, and how they interact with each other. This design ensures the system meets its functional and non-functional requirements, such as scalability, security, and performance.

The system is organized into multiple layers and modules, each serving a specific purpose:

- User Layer:

  - Mobile App: The mobile app interfaces with the system, providing users with a means to control and monitor their air conditioners. The app communicates with the Gateway via MQTT to send commands and receive real-time updates.

- Gateway Layer:

  - Gateway Device: The master unit in the system that manages communication with the Nodes via BLE Mesh and connects to the cloud via LTE for remote access. Every Gwy device is identified by its unique <GwySerNo>. This is set during flashing the Gwy device and cannot be changed later. (Eg. GWY00001)

  - MQTT Client: The Gateway connects to an MQTT broker, enabling bidirectional communication with the mobile app.

  - IR Software module: Uses a third-party IR library to decode and transmit IR signals for controlling AC units.

- Node Layer:

  - Node Device: A BLE Mesh device that acts as a slave, responsible for controlling the AC based on commands received from the Gateway. It reports status information back to the Gateway and executes local control functions when necessary. Every Gwy device is identified by its unique <NodeSerNo>. This is set during flashing the Gwy device and cannot be changed later. (Eg. N00001).

- Communication Layer:

  - BLE Mesh: Provides a robust, low-power communication network between the Gateway and Node devices. BLE Mesh ensures the scalability and reliability of communication in environments with multiple devices.

  - LTE & MQTT: The Gateway communicates with the MQTT broker over the LTE network, enabling cloud connectivity, remote status updates, and command execution.

- Cloud Layer:

  - Cloud Platform: Hosts the MQTT broker and processes the data sent by the Gateway. This layer is responsible for long-term storage, analytics, and any external interactions

- Flash Memory Layer:

  - Non-Volatile Storage: Provides the ability to store contents in flash memory to preserve them across power cycles

## Key Interactions and Data Flow

- Mobile App to MQTT Broker: The mobile app communicates with the MQTT broker to send control commands to the Gateway.

- Gateway to Nodes: The Gateway manages and controls the Nodes through BLE Mesh, sending commands and receiving status updates.

- Gateway to Cloud: The Gateway updates the cloud platform with status data, device health information, via MQTT.

- IR Control: When the Gateway sends a command to control the AC, the IR Software module translates the MQTT command into the appropriate IR signals and transmits them to the AC unit.

This architecture ensures a robust, scalable, and secure system for managing air conditioning units, providing real-time control, monitoring, and seamless integration with external cloud services. The system is designed to handle a growing number of devices, ensuring reliability and performance even as the network expands.

# Limitations & Assumptions

## Limitations

- Connectivity Range: The BLE mesh network’s effective range may be limited by environmental factors such as physical obstructions or interference from other wireless devices.

- Internet Connectivity: While the Gateway has LTE connectivity, the system’s remote control and status updates depend on the availability of a stable internet connection. Due to the nature of MQTT and potential delays in cloud communication, there may be minor latencies in status updates or command execution, especially during high network traffic.

- IR Library Support: The system uses a third-party IR library for decoding IR signals and transmitting AC control IR commands. As the library is still under continuous development, not all AC models will be supported. Additionally, within the same brand, different models may use different IR protocols, meaning the model is more important than the brand when ensuring compatibility. When talking about support, we need to consider two things: Ability to Decode IR Protocol and ability send IR Protocol.

- Teaching Mode: Since Teaching Mode works by capturing and storing the IR signal RAW values and replaying them back to control an AC, only a maximum of 16 commands can be stored currently. (Power off command, Temperature 18 to Temperature 32 commands (with Power ON)). Which means, when device uses teaching mode, it will be able to control only Power and Temperature.

## Decodeable Protocols

Below table represents the list of protocols that are decodable, meaning recognizable by the devices (Gwy and Node). A protocol may be decodable, that doesn’t mean the device can send an IR command using that protocol. In this case, teaching mode with error checking must be used for Configuration.

| Brand | Protocol |
| --- | --- |
| Carrier | Carrier |
|  | Carrier40 |
|  | Carrier64 |
|  | Carrier84 |
|  | Carrier128 |
| Daikin | Daikin |
|  | Daikin2 |
|  | Daikin64 |
|  | Daikin128 |
|  | Daikin152 |
|  | Daikin160 |
|  | Daikin176 |
|  | Daikin200 |
|  | Daikin216 |
|  | Daikin280 |
|  | Daikin312 |
| Haier | Haier |
|  | HaierYRW02 |
|  | Haier160 |
|  | Haier176 |
| Hitachi | Hitachi1 |
|  | Hitachi2 |
|  | Hitachi3 |
|  | Hitachi264 |
|  | Hitachi296 |
|  | Hitachi344 |
|  | Hitachi424 |
| LG | LG |
|  | LG2 |
| Mitsubishi | Mitsubishi |
|  | Mitubishi1 |
|  | MitsubishiAC |
|  | Mitubishi112 |
|  | Mitsubishi136 |
|  | MitsubishiHeavy |
| Samsung | Samsung |
|  | SamsungAC |
|  | Samsung36 |
| Toshiba | Toshiba56 |
|  | Toshiba72 |
|  | Toshiba80 |
| Voltas | Voltas |

## Sendable Protocols

Below table represents the list of protocols that the devices could send an IR command in. It also provides the information as to what functionalities are supported by the library in each of these protocols.

| Protocol | Power | Temp | Fan | Mode | SwingH | SwingV | OnTimer | OffTimer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier64 | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes |
| Daikin2 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Daikin64 | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes |
| Daikin128 | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes |
| Daikin152 | Yes | Yes | Yes | Yes | No | Yes | No | No |
| Daikin160 | Yes | Yes | Yes | Yes | No | Yes | No | No |
| Daikin176 | Yes | Yes | Yes | Yes | Yes | No | No | No |
| Daikin200 | Yes | Yes | Yes | Yes | Yes | No | No | No |
| Daikin216 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Daikin280 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Haier | No | Yes | Yes | Yes | No | Yes | No | No |
| Haier160 | Yes | Yes | Yes | Yes | No | Yes | Yes | Yes |
| Haier176 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Hitachi | Yes | Yes | Yes | Yes | Yes | Yes | No | No |
| Hitachi1 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Hitachi296 | Yes | Yes | Yes | Yes | No | No | No | No |
| Hitachi424 | Yes | Yes | Yes | Yes | No | Yes | No | No |
| LG | Yes | Yes | Yes | Yes | Yes | Yes | No | No |
| Mitsubishi | Yes | Yes | Yes | Yes | No | No | No | No |
| Mitsubishi88 | Yes | Yes | Yes | Yes | Yes | Yes | No | No |
| Mitsubishi112 | Yes | Yes | Yes | Yes | No | Yes | No | No |
| Mitsubishi136 | Yes | Yes | Yes | Yes | No | Yes | No | No |
| Mitsubishi152 | Yes | Yes | Yes | Yes | Yes | Yes | No | No |
| Samsung | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Toshiba | Yes | Yes | Yes | Yes | No | Yes | No | No |
| Voltas | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

## Assumptions

- Network Stability: It is assumed that the Gateway will always have access to a reliable LTE network for remote communication and MQTT-based control.

- IR noise free environment: During initial setup and during the process of Teaching, the devices must be in an IR noise free environment.

# Physical Button Functionalities

| Press type | Gateway | Node |
| --- | --- | --- |
| Single press | None | None |
| Double press | Initiate Power Cycle sequence | Initiate Power Cycle sequence |
| Triple press | Unregisters device | Unprovisions device |
| Long press (1-3s) | Enter/Exit Teaching mode | Enter/Exit Teaching mode |
| Long press (>3s) | Factory Reset Device | Factory Reset Device |

# LED Indications

| Indication | Gateway | Node |
| --- | --- | --- |
| Solid Red | Device not connected to MQTT | Device is Unprovisioned and non-connectable |
| Toggling Red and Blue | Device is Unregistered | Device in Unprovisioned and connectable |
| Solid Blue | Device is Unconfigured | Device is Unconfigured |
| Solid Green | Fully operational | Fully operational |
| Blinking Blue | Device in Teaching mode | Device in Teaching mode |
| Blinking Yellow | Device in power cycle sequence | Device in power cycle sequence |
| Blinking Purple | Sending out IR signal | Sending out IR signal |
| Blinking Red | Invalid event/command detected | Invalid event/command detected |
| Blinking Green | Device detected IR Signal | Device detected IR Signal |

# JSON Packets

| Packet ID | Packet Name |
| --- | --- |
| 0 | Gwy Registration Packet |
| 1 | Gwy AC Remote Configuration Packet |
| 2 | Gwy Unregistration Packet |
| 3 | Gwy AC Control Packet |
| 4 | Gwy Manual AC Control ACK |
| 5 | Gwy AC Remote Reconfiguration Packet |
| 6 | Gwy Heartbeat ACK |
| 7 | Gwy Heartbeat Publish Configuration Packet |
| 8 | Gwy Teaching Mode Packet |
| 9 | Gwy Teaching Mode Command Selection Packet |
| 10 | Gwy Debug Info Packet |
| 12 | Gwy General ACK |
| 100 | Node Provisioning Packet |
| 101 | Node AC Remote Configuration ACK |
| 102 | Node Unprovisioning Packet |
| 103 | Node AC Control Packet |
| 104 | Node Manual AC Control ACK |
| 105 | Node AC Remote Reconfiguration Packet |
| 106 | Node Heartbeat ACK |
| 107 | Node Heartbeat Publish Configuration Packet |
| 108 | Node Teaching Mode Packet |
| 109 | Node Teaching Mode Command Selection Packet |
| 110 | Node Debug Info Packet |
| 112 | Node General ACK |

For more information about these packets, what parameters are required, the data type of values and the value range, please refer to this link. This will contain examples of these packets as well.

# Error Codes

| Error Code | Error Code Name |
| --- | --- |
| -1 | FAILURE |
| 0 | SUCCESS |
| 1 | JSON_PACKET_INVALID |
| 2 | MISSING_PACKET_ID |
| 3 | PACKET_ID_EXCEEDING_RANGE |
| 4 | MISSING_MSG_SEQ_NO |
| 5 | MSG_SEQ_NO_EXCEEDING_RANGE |
| 6 | MISSING_LOCATION |
| 7 | LOCATION_EXCEEDING_RANGE |
| 8 | NODE_COMM_TIMEOUT |
| 9 | GWY_NOT_REG |
| 10 | GWY_ALREADY_REG |
| 11 | MISSING_MACID |
| 12 | INVALID_MACID |
| 13 | MISSING_ELEMENT_ADDR |
| 14 | ELEMENT_ADDR_EXCEEDING_RANGE |
| 15 | DEVICE_NOT_CONFIGURED_WITH_AC_REMOTE |
| 16 | MISSING_POWER |
| 17 | POWER_EXCEEDING_RANGE |
| 18 | MISSING_MODE |
| 19 | MODE_EXCEEDING_RANGE |
| 20 | MISSING_FAN_SPEED |
| 21 | FAN_SPEED_EXCEEDING_RANGE |
| 22 | MISSING_TEMPERATURE |
| 23 | TEMPERATURE_EXCEEDING_RANGE |
| 24 | MISSING_SWINGH |
| 25 | SWINGH_EXCEEDING_RANGE |
| 26 | MISSING_SWINGV |
| 27 | SWINGV_EXCEEDING_RANGE |
| 28 | MISSING_ONTIMER |
| 29 | ONTIMER_EXCEEDING_RANGE |
| 30 | MISSING_OFFTIMER |
| 31 | OFFTIMER_EXCEEDING_RANGE |
| 32 | MISSING_LOCKING |
| 33 | LOCKING_EXCEEDING_RANGE |
| 34 | MISSING_TEMPERATURE_UPPER_LIMIT |
| 35 | TEMPERATURE_UPPER_LIMIT_EXCEEDING_RANGE |
| 36 | MISSING_TEMPERATURE_LOWER_LIMIT |
| 37 | TEMPERATURE_LOWER_LIMIT_EXCEEDING_RANGE |
| 38 | INVALID_TEMPERATURE_LOCKING_LIMITS |
| 39 | MISSING_PUBLISH_PERIOD |
| 40 | PUBLISH_PERIOD_EXCEEDING_RANGE |
| 41 | MISSING_RESET_DEVICE |
| 42 | RESET_DEVICE_EXCEEDING_RANGE |
| 43 | MISSING_LOGGING |
| 44 | LOGGING_EXCEEDING_RANGE |
| 45 | IR_PROTOCOL_DECODEABLE_ONLY |
| 46 | IR_PROTOCOL_FULLY_UNSUPPORTED |
| 47 | MISSING_ERROR_CHECK |
| 48 | ERROR_CHECK_EXCEEDING_RANGE |
| 49 | MISSING_TEACHING_START |
| 50 | TEACHING_START_EXCEEDING_RANGE |
| 51 | MISSING_STARTING_TEMPERATURE |
| 52 | STARTING_TEMPERATURE_EXCEEDING_RANGE |
| 53 | MISSING_ENDING_TEMPERATURE |
| 54 | ENDING_TEMPERATURE_EXCEEDING_RANGE |
| 55 | STARTING_TEMPERATURE_LESS_THAN_ENDING_TEMPERATURE |
| 56 | ENTERED_TEACHING_MODE |
| 57 | EXITED_TEACHING_MODE |
| 58 | DEVICE_ALREADY_IN_TEACHING_MODE |
| 59 | DEVICE_NOT_IN_TEACHING_MODE |
| 60 | POWER_NOT_AVAILABLE_IN_IR_SIGNAL_DECODED_STRING |
| 61 | MODE_NOT_AVAILABLE_IN_IR_SIGNAL_DECODED_STRING |
| 62 | FANSPEED_NOT_AVAILABLE_IN_IR_SIGNAL_DECODED_STRING |
| 63 | TEMPERATURE_NOT_AVAILABLE_IN_IR_SIGNAL_DECODED_STRING |
| 64 | IR_CMD_NOT_AVAILABLE_IN_FLASH |
| 65 | NODE_NOT_FOUND_IN_PROVISIONER_DATABASE |
| 66 | NODE_NOT_PROVISIONED |
| 67 | IR_TASK_CREATION_FAILED (QMAX ‘s Internal Use only) |
| 68 | ENQUEUING_INTO_COMMAND_QUEUE_FAILED |
| 69 | DEVICE_DATA_ERASURE_SUCCESSFUL |
| 70 | DEVICE_DATA_ERASURE_FAILED |
| 71 | IR_PARTITION_INIT_FAILED (QMAX ‘s Internal Use only) |
| 72 | POWER_INVALID_FORMAT |
| 73 | TEMPERATURE_INVALID_FORMAT |
| 74 | FAN_SPEED_INVALID_FORMAT |
| 75 | MODE_INVALID_FORMAT |
| 76 | SWINGH_INVALID_FORMAT |
| 77 | SWINGV_INVALID_FORMAT |
| 78 | LOCKING_INVALID_FORMAT |
| 79 | ONTIMER_INVALID_FORMAT |
| 80 | OFFTIMER_INVALID_FORMAT |
| 81 | TEMPERATURE_UPPER_LIMIT_INVALID_FORMAT |
| 82 | TEMPERATURE_LOWER_LIMIT_INVALID_FORMAT |
| 83 | PACKETID_INVALID_FORMAT |
| 84 | MSG_SEQ_NO_INVALID_FORMAT |
| 85 | ELEMENT_ADDR_INVALID_FORMAT |
| 86 | LOCATION_INVALID_FORMAT |
| 87 | PUBLISH_PERIOD_INVALID_FORMAT |
| 88 | MACID_INVALID_FORMAT |
| 89 | TEACHING_START_INVALID_FORMAT |
| 90 | STARTING_TEMPERATURE_INVALID_FORMAT |
| 91 | ENDING_TEMPERATURE_INVALID_FORMAT |
| 92 | RESTART_INVALID_FORMAT |
| 93 | RESET_INVALID_FORMAT |

Most of the above-mentioned error codes are self-explanatory as to what they mean and when they will occur. Some of the ones are explained below:

- Error codes containing “MISSING” mean, a JSON packet was received by the Gwy without a required parameter in it.

- Error codes containing “EXCEEDING_RANGE” mean, a JSON packet was received by the Gwy with one of the parameters containing a value out of range.

# MQTT Topics

Gwy uses the following topics for communication with the MQTT Broker.

| Topic | Usage |
| --- | --- |
| <GwySerNo>/command | Gwy receives commands from this topic |
| <GwySerNo>/message | Gwy sends out ACKs to this topic |
| <GwySerNo>/alive | Gwy sends a message to this topic once every minute to keep the MQTT connection alive |
| GWYS/will | All Gwy devices will make a will configuration to this topic. Meaning, when Gwy loses its MQTT connection, the MQTT broker, on behalf of the Gwy will send out a message “<GwySerNo> disconnected Unexpectedly” to the clients that subscribed to this topic |

# Teaching Mode

Teaching Mode is a method in which IR commands fired from the AC remote is captured and stored by the devices (Gwy and Node) and then replayed back to control the AC. Teaching mode is intended to be used when the IR protocol used by the AC Remote is not in the list of sendable protocols.

Teaching Mode comes off as two variants.

- Teaching Mode with Error Check

- Teaching Mode without Error Check

## Teaching Mode with Error Check

If the IR Protocol used by the AC remote is not in the list of sendable protocols, but is in the list of decodable protocols, then Teaching Mode with Error Check will be used by the device. What this means is that the device will have the ability to know whether the correct IR command as per the Teaching mode command sequence was received by the device. For example, the first IR command to be received by the device is “Power OFF” command. But if the user accidentally sends a different command, then the device will not store it and will send an ack with appropriate error code.

## Teaching Mode without Error Check

If the IR Protocol used by the AC remote is neither sendable nor decodable, then Teaching Mode without Error check will be used by the device. In this case, to make the process reliable and robust, the user will have to choose the command that he’s going to send from the remote and then send the command. For example, the user can choose “Power OFF” command and then fire the Power OFF command from the AC remote. By this way, the device will store the command in the right slot. Even if any error occurs, the user can re-choose the slot and overwrite the wrongly stored command with the right one. The device will automatically exit the teaching mode, once the necessary number of commands have been received.

## Teaching Mode command sequence

| Command Number | Power | Temperature |
| --- | --- | --- |
| 1 | Off | Anything |
| 2 | On | 18 |
| 3 | On | 19 |
| 4 | On | 20 |
| 5 | On | 21 |
| 6 | On | 22 |
| 7 | On | 23 |
| 8 | On | 24 |
| 9 | On | 25 |
| 10 | On | 26 |
| 11 | On | 27 |
| 12 | On | 28 |
| 13 | On | 29 |
| 14 | On | 30 |
| 15 | On | 31 |
| 16 | On | 32 |

# Setup Instructions

This section contains step by step instructions on how to set up devices according to different configurations.

## One Gwy – Normal Mode

- Power ON Gateway

- Register Gateway

- If Gateway LED is solid Blue, Move on to next step.

- Configure the Gateway with AC remote by pointing the AC remote towards the Gateway and pressing any button (preferably the Power ON/OFF button)

- If Gateway LED is solid Green, Gateway is now ready to control the AC.

## One Gwy – Teaching Mode with Error Check enabled

  - Power ON Gateway

  - Register Gateway

  - If Gateway LED is solid Blue, Move on to next step.

  - Configure the Gateway with AC remote by pointing the AC remote towards the Gateway and pressing any button (preferably the Power ON/OFF button)

  - The Gateway will check if the IR Protocol used by the AC remote is in the list of the Sendable Protocols. In this case, assume it’s not in the list. Then, the next step is to proceed with Teaching Mode.

  - Send Gateway Teaching Mode packet.

  - Gateway Enters Teaching Mode with Error Check enabled.

  - Fire IR commands in the right sequence. Pay attention to LED indication and Ack received by the MQTT Broker to know if the command was successful

  - Once the necessary number of commands gets received, the Gateway will automatically quit teaching mode and get configured.

  - If the Gateway is solid Green, Gateway is now ready to control the AC.