# Scheduling

Unimation wants to add a feature called “Scheduling”. Unimation intends this feature to have the following capabilities:

- Set Schedules with the ability to customize the following parameters:

  - Power

  - Temperature

  - Fan Speed

  - Mode

  - Time of Schedule (up to minute level precision (not seconds))

  - Day of Schedule

- Set more than one schedule in a day

- Scheduled command should be fired out even without an internet connection.

- Schedules need to be stored in non-volatile memory to survive power cycles.

- If device was not in power ON state during a schedule, and hence missed to fire that schedule command, then that scheduled command needs to be fired as soon as the device comes back ON.

# MQTT Packets required

To accommodate the Scheduling feature, the following new MQTT packets and ACKS need to be defined:

Add Schedule packet

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 11 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2-65535] |
|  | “Power” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “Mode” | : | “Cool” | // (String) | [“Cool“, “Dry“, “Auto“, “Fan“, “Hot“] |
|  | “Temperature” | : | 25 | // (Number) | [18 - 32] |
|  | “FanSpeed” | : | 2 | // (Number) | [1 - 5] |
|  | “Hour” | : | 20 | // (Number) | [0 - 23] |
|  | “Minute” | : | 36 | // (Number) | [0 - 59] |
|  | “Day” | : | 6 | // (Number) | [0 = Sunday, 6 = Saturday], [0 - 6] |
| } |  |  |  |  |  |

The Add Schedule ACK will contain the above parameters and two more parameters: Error code, ScheduleID.

Remove Schedule Packet

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 11 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “ScheduleID” | : | 56 | // (Number) | [0 – 69] |
| } |  |  |  |  |  |

The Remove Schedule ACK will contain the above parameters and one more parameter: Error Code

- Enable/Disable Schedule (Optional)

This will allow to enable/disable a schedule instead of deleting based on ScheduleID.

Technical Details

- Number of Bytes of Non-Volatile storage required per schedule = 9 Bytes

- EEPROM storage availability (Before Teaching Mode usage) = 32768 Bytes

- EEPROM storage available (After Teaching Mode usage) = (Approx.) 25068 Bytes (Teaching Mode needs to store 11 commands, considering one command to take 700 bytes)

Things to consider

- Polling strategy to get current local time?

  - Polling current time by directly querying LTE will increase traffic and reduces system performance

  - Instead, we can get current time at device bootup and maintain an ESP timer.

- Prevent conflicting schedules?