July 22, 2024

BLE Mesh based IR AC Controller

Software Design Document

Table of Contents

1	Introduction	6

2	Purpose of Document	6

3	Product Brief	6

4	Software Architecture	6

5	Basics and Terminologies	7

5.1	MQTT Broker	7

5.2	Gateway	7

5.3	Registration	7

5.4	Unregistration	7

5.5	Node	7

5.6	Provisioning	7

5.7	Unprovisioning	7

5.8	AC Remote Configuration	8

5.9	Teaching Mode	8

5.10	List of AC Brands Supported by Gwy/Node	9

5.11	MQTT Publishing	9

5.12	MQTT Subscribing	9

6	Gateway LED Indications	9

7	Node LED Indications	10

8	Gateway Button Press Logics	10

9	Node Button Press Logics	11

10	MQTT Communication Packets	11

10.1	Error codes	12

10.1.1	FAILURE	14

10.1.2	SUCCESS	14

10.1.3	JSON_PACKET_ID_NOT_FOUND	14

10.1.4	JSON_PACKET_ID_UNKNOWN	14

10.1.5	MSG_SEQ_NO_NOT_FOUND	14

10.1.6	GWY_SER_NO_NOT_FOUND	15

10.1.7	GWY_SER_NO_INVALID	15

10.1.8	LOCATION_NOT_FOUND	15

10.1.9	LOCATION_EXCEEDING_RANGE	15

10.1.10	NODE_COMM_TIMEOUT	15

10.1.11	GWY_NOT_REG	15

10.1.12	GWY_ALREADY_REG	15

10.1.13	MAC_ID_NOT_FOUND	16

10.1.14	MAC_ID_CONTAINS_INVALID_CHARS_OR_INVALID_FORMAT	16

10.1.15	INVALID_MAC_ID_LENGTH	16

10.1.16	NODE_SER_NO_NOT_FOUND	16

10.1.17	ELEMENT_ADDR_NOT_FOUND	16

10.1.18	GWY_NOT_CONFIGURED_WITH_AC_REMOTE	16

10.1.19	NODE_NOT_CONFIGURED_WITH_AC_REMOTE	16

10.1.20	POWER_NOT_FOUND	16

10.1.21	POWER_EXCEEDING_RANGE	17

10.1.22	MODE_NOT_FOUND	17

10.1.23	MODE_EXCEEDING_RANGE	17

10.1.24	FAN_SPEED_NOT_FOUND	17

10.1.25	FANSPEED_EXCEEDING_RANGE	17

10.1.26	TEMPERATURE_NOT_FOUND	17

10.1.27	TEMPERATURE_EXCEEDING_RANGE	17

10.1.28	SWING_H_NOT_FOUND	17

10.1.29	SWING_H_EXCEEDING_RANGE	18

10.1.30	SWING_V_NOT_FOUND	18

10.1.31	SWING_V_EXCEEDING_RANGE	18

10.1.32	ONTIMER_NOT_FOUND	18

10.1.33	ONTIMER_EXCEEDING_RANGE	18

10.1.34	OFFTIMER_NOT_FOUND	18

10.1.35	OFFTIMER_EXCEEDING_RANGE	18

10.1.36	LOCKING_NOT_FOUND	18

10.1.37	LOCKING_EXCEEDING_RANGE	18

10.1.38	TEMP_LOCK_UP_LIMIT_NOT_FOUND	18

10.1.39	TEMP_LOCK_LOW_LIMIT_NOT_FOUND	19

10.1.40	TEMP_LOCK_UP_LIMIT_EXCEEDS_ABS_TEMP_UP_LIMIT	19

10.1.41	TEMP_LOCK_LOW_LIMIT_EXCEEDS_ABS_TEMP_LOW_LIMIT	19

10.1.42	ILLOGICAL_LOCKING_TEMP_LIMIT	19

10.1.43	PUBLISH_PERIOD_NOT_FOUND	19

10.1.44	PUBLISH_PERIOD_EXCEEDS_RANGE	19

10.1.45	MSG_SEQ_NO_EXCEEDING_RANGE	19

10.1.46	NODE_SER_NO_INVALID	19

10.1.47	RESET_DEVICE_NOT_FOUND	19

10.1.48	LOGGING_FLAG_NOT_FOUND	20

10.1.49	FORBIDDEN_OPERATION	20

10.1.50	UNKNOWN ERROR CODE	20

10.1.51	AC_REMOTE_UNSUPPORTED	20

10.1.52	JSON_PACKET_INVALID	20

10.2	Common JSON parameters	20

10.3	Gateway Registration Packet	21

10.4	Gateway Registration ACK	22

10.5	Gateway AC Remote Configuration ACK	23

10.6	Gateway Unregistration Packet	24

10.7	Gateway Unregistration ACK	25

10.8	Gateway AC Control Packet	26

10.9	Gateway AC Control ACK	27

10.10	Gateway AC Manual control ACK	29

10.11	Gateway AC Remote Reconfiguration Packet	31

10.12	Gateway AC Remote Reconfiguration ACK	32

10.13	Gateway Heartbeat ACK	33

10.14	Gateway Heartbeat Publish Configuration Packet	34

10.15	Gateway Heartbeat Publish Configuration ACK	35

10.16	Gateway Teaching Mode Start Packet	36

10.17	Gateway Teaching Mode Start ACK	38

10.18	Gateway Teaching Mode End ACK	39

10.19	Gateway Debug Info Packet (Only for QMAX Internal use)	40

10.20	Gateway Debug Info ACK (Only for QMAX Internal use)	41

10.21	Node Provision Packet	42

10.22	Node Provision ACK	43

10.23	Node AC Remote Configuration ACK	44

10.24	Node Unprovision Packet	45

10.25	Node Unprovision ACK	46

10.26	Node AC Control Packet	47

10.27	Node AC Control ACK	49

10.28	Node AC Manual control ACK	50

10.29	Node AC Remote Reconfiguration Packet	52

10.30	Node AC Remote Reconfiguration ACK	53

10.31	Node Heartbeat ACK	54

10.32	Node Heartbeat Publish Configuration Packet	55

10.33	Node Heartbeat Publish Configuration ACK	56

10.34	Node Teaching Mode Start Packet	57

10.35	Node Teaching Mode Start ACK	59

10.36	Node Teaching Mode End ACK	60

10.37	Node Debug Info Packet (Only for QMAX Internal use)	61

10.38	Node Debug Info ACK (Only for QMAX Internal use)	62

Table of Figures

Figure 4.1 Software Architecture Block Diagram	6

Figure 10.1 Gateway Registration process flow	21

Figure 10.2 Gateway Registration process flow	22

Figure 10.3 Gateway Configuration process flow	23

Figure 10.4 Gateway Unregistration Process flow	24

Figure 10.5 Gateway Unregistration Process Flow	25

Figure 10.6 Gateway AC Control Process flow	27

Figure 10.7 Gateway AC Control Process flow	28

Figure 10.8 Gateway AC manual control Process flow	30

Figure 10.9 Gateway AC Remote Reconfiguration Process Flow	31

Figure 10.10 Gateway AC Remote Reconfiguration Process flow	32

Figure 10.11 Gateway Heartbeat ACK Process flow	33

Figure 10.12 Gateway Heartbeat Publish Configuration Process flow	34

Figure 10.13 Gateway Heartbeat Publish Configuration ACK Process flow	35

Figure 10.16 Gateway Teaching Mode Start Flow	36

Figure 10.17 Gateway Teaching Mode Start ACK Flow	38

Figure 10.18 Gateway Teaching Mode End ACK Flow	39

Figure 10.19 Gateway Debug Info Process Flow	40

Figure 10.20 Gateway Debug Info ACK flow	41

Figure 10.21 Node Provisioning Flow	42

Figure 10.22 Node Provisioning ACK Flow	43

Figure 10.23 Node Configuration Process flow	44

Figure 10.24 Node Unprovsioning Process flow	45

Figure 10.25 Node Unprovisioning Process flow	46

Figure 10.26 Node AC Control Process flow	48

Figure 10.27 Node AC Control Process flow	49

Figure 10.28 Node AC Locking Process flow	51

Figure 10.29 Node Reconfiguration Process flow	52

Figure 10.30 Node Reconfiguration Process flow	53

Figure 10.31 Node Heartbeat Process flow	54

Figure 10.32 Node Publish Configuration process flow	55

Figure 10.33 Node Publish Configuration process flow	56

Figure 10.34 Node Teaching Mode Start Flow	57

Figure 10.35 Node Teaching Mode Start ACK flow	59

Figure 10.36 Node Teaching Mode End flow	60

Figure 10.37 Node Debug Info Process Flow	61

Figure 10.38 Node Debug Info ACK flow	62

List of Tables

Table 5.1 – AC control parameters for Supported protocol	8

Table 5.2 – List of Supported AC Brands	9

Table 6.1 Gateway LED Indication	9

Table 7.1 Node LED Indication	10

Table 8.1 Gateway Button Press Logic	10

Table 9.1 Node Button Press Logics	11

Table 10.1 MQTT Communication Packets	11

Table 10.2 Error Codes	12

Table 10.3 Common JSON Parameter description	20

Table 10.4 AC Control Parameters	26

Table 10.5 AC Control Parameters	47

Revision History

| Version | Release Date | Prepared by | Remarks |
| --- | --- | --- | --- |
| 0.2 | 01.04.2024 | Kulasekaran | - Draft Version |
| 0.3 | 10.04.2024 | Kulasekaran | - Removed the Heartbeat related content as temperature data has been decided to be taken as heartbeat - Added Teaching mode packet and ACK for Gwy and Node |
| 0.4 | 24.04.2024 | Kulasekaran | - Changed the Temperature upper and lower limit keys |
| 0.5 | 24.06.2024 | Kulasekaran | - Added Solid Purple indication for when sending out IR command in both Gwy and Node - Added packet information regarding Teaching modes - Added Boot up LED pattern information for both Gwy and Node in the LED Table - Modified Button press logics - Added Debug Info packets for Gwy and Node - Changed ErrorCode values - Minor changes with more clear descriptions to certain sections across the whole document - Added Detailed Error Code information - LED indication change for Teaching mode and AC Remote configuration mode. |
| 0.6 | 30.07.2024 | Kulasekaran | - Added ResetDevice and Logging to the Error code - Added Unsupported Remote LED indication - Removed NODE_ALREADY_PROV error code - Added AC_REMOTE_UNSUPPORTED error code - The default Heartbeat interval increased from 10s to 300s. - Added JSON_PACKET_INVALID error code |
| 0.7 | 01.08.2024 | Kulasekaran |  |

Acronyms and Abbreviations

| MQTT | Message Queuing Telemetry Transport |
| --- | --- |
| BLE | Bluetooth Low Energy |
| UART | Universal Asynchronous Receiver / Transmitter |
| LTE | Long-term evolution (4G) |
| GWY | Gateway |
| IR | Infrared |
| ACK | Acknowledgement |

# Introduction

The goal of Unimation Robotics is to create a Bluetooth mesh system that will allow for remote air conditioner operation. Prototypes that meet Unimation Robotics' specifications will be designed, developed, tested, and delivered by QMAX Systems India Pvt. Ltd., an electronics engineering and R&D services company.

# Purpose of Document

This document aims to provide an overview of the software design specifics of the Unimation Robotics product being developed by QMAX Systems India Pvt. Ltd.

# Product Brief

The overall product mainly focuses on building a BLE Mesh based system consisting of two types of devices: 1. Gateway 2. Node. Both the devices can control Air conditioners by transmitting IR signals using the IR transmitter on-board. Also, both the devices can detect IR signals using the IR receiver on-board. Adding to that, both the devices have temperature sensors on-board, that help in tracking the ambient temperature and do logical operation based on requirement. Gateway is master type device and Node is a slave type device. Gateways receive commands from cloud and perform necessary operation or relay them to the appropriate node to perform the necessary task. Nodes after performing the task will relay back acknowledgement to Gateway, which will be relayed back to Cloud.

# Software Architecture

The following image represents the Software architecture of the whole system and how the devices communicate with each other.

Figure 4.1 Software Architecture Block Diagram

# Basics and Terminologies

BLE Mesh based systems involve a few initial steps to be followed to add/remove devices into/from the network. The following section provides information reg. what those steps/processes are.

## MQTT Broker

MQTT Broker acts as the backend system which coordinates messages between the different MQTT clients.

## Gateway

Gateway is a type of device in the BLE Mesh network which acts as a mediator between the slave devices (Nodes) and the MQTT Broker. Each Gateway is identified with the BLE Mesh network by their unique serial number. In our application, Gateways also take the role as BLE Mesh Provisioner, which means, they can add Nodes into the BLE Mesh network and remove Nodes from the BLE Mesh network Gateways can’t communicate among themselves.

## Registration

Registration is the process of adding a Gateway device into the BLE Mesh network.

## Unregistration

Unregistration is the process of removing a Gateway device from the BLE Mesh network.

## Node

Node is a type of device in the BLE Mesh network which acts as slaves. Typically, Nodes on a BLE Mesh network don’t communicate with the MQTT Broker directly. They will communicate via Gateway to the MQTT Broker. But, since it’s a mesh network, Nodes can communicate to Gateways out of their communication range by the concept of relaying the message through a chain of nodes that will reach the Gateway.

## Provisioning

Provisioning is the process of adding a Node device into the BLE Mesh network.

## Unprovisioning

Unprovisioning is the process of removing a Node device from the BLE Mesh network.

## AC Remote Configuration

This procedure is shared by Node and Gwy. It is necessary to complete this procedure before attempting to operate an air conditioner. A Gwy/Node will identify which AC remote it will function as through this configuration process. See Gateway AC Remote Configuration ACK or Node AC Remote Configuration ACK for further details on this.

## Teaching Mode

Teaching mode is a process, in which a sequence of IR signals is sent out from the Physical AC remote, recorded on Gwy/Node and made to be replayed back on-command to control the ACs. For more information regarding Teaching Mode, look at Gateway Teaching Mode Start Packet or Node Teaching Mode Start Packet.

Teaching mode makes it possible for Gwy/Node to be able to only switch ON/OFF or Set Temperature for unsupported AC protocols. For supported AC protocols, the Gwy/Node will be able to control the following parameters displayed in Table 5.1, provided it is supported by the AC in the first place. Look at Table 5.2 for the list of supported AC Brands.

Table 5.1 – AC control parameters for Supported protocol

| Sl. No | Parameter |
| --- | --- |
| 1 | Power |
| 2 | Temperature |
| 3 | FanSpeed |
| 4 | Mode |
| 5 | Swing Horizontal |
| 6 | Swing Vertical |
| 7 | On Timer |
| 8 | Off Timer |

## List of AC Brands Supported by Gwy/Node

Note: An AC manufacturer may have different AC protocols for different models of their Air conditioners. So, it’s not that the Gwy/Node will be able to contain any model of Air conditioner from the list of brands listed in Table 5.2 below. If an AC remote is unsupported, then the device will emit blinking RED LED indication and also an ACK with AC_REMOTE_UNSUPPORTED error code is sent back.

Table 5.2 – List of Supported AC Brands

| Sl. No | AC Brand |
| --- | --- |
| 1 | Daikin |
| 2 | LG |
| 3 | Voltas |
| 4 | Samsung |
| 5 | Hitachi |
| 6 | Haier |
| 7 | Carrier |
| 8 | Toshiba |
| 9 | Mitsubishi |

## MQTT Publishing

In MQTT, publishing is the concept of sending data to a specific topic. Topics are like channels in MQTT. There can be any number of topics and all topics are unique. Devices that have subscribed to the topic will receive data from that topic.

## MQTT Subscribing

In MQTT, Subscribing is the concept of listening for data from a specific topic.

# Gateway LED Indications

Table 6.1 Gateway LED Indication

| Sl. No | Pattern | Description |
| --- | --- | --- |
| 1 | Toggle Red + Green + Blue | Gateway has booted successfully. The LED will stay in this pattern for 3s and then resume normal operation and will be in one of the states below. |
| 2 | Solid Red | Gateway is not connected with MQTT server |
| 3 | Toggling Red + Blue (500ms Red ON, Blue OFF ¦ 500ms Red OFF, Blue ON) | Gateway is not registered |
| 4 | Solid BLUE | Gateway is in AC remote configuration mode |
| 5 | Blinking BLUE | Gateway is in Teaching mode |
| 6 | Solid Green | Gateway is connected to MQTT, Registered and Configured. Meaning, the Gateway is fully operational |
| 7 | Solid Purple | Gateway is sending out an IR command. This is momentary. This will last less than a second. |
| 8 | Blinking RED (200ms ON ¦ 200ms OFF) x3 | Unsupported remote detected during AC Remote Configuration process. |

# Node LED Indications

Table 7.1 Node LED Indication

| Sl. No | Parameter | Description |
| --- | --- | --- |
| 1 | Toggle Red + Green + Blue | Node has booted successfully. The LED will stay in this pattern for 3s and then resume normal operation and will be in one of the states below. |
| 2 | Toggling Red + Blue (500ms Red ON, Blue OFF ¦ 500ms Red OFF, Blue ON) | Node is not Provisioned |
| 3 | Solid BLUE | Node is in AC remote configuration mode |
| 4 | Blinking BLUE | Node is in Teaching mode |
| 5 | Solid Green | The node is provisioned and configured. Meaning, the node is fully operational |
| 6 | Solid Purple | Gateway is sending out an IR command. This is momentary. This will last less than a second. |
| 7 | Blinking RED (200ms ON ¦ 200ms OFF) x3 | Unsupported remote detected during AC Remote Configuration process. |

# Gateway Button Press Logics

Table 8.1 Gateway Button Press Logic

| Sl. No | Button Press type | Action |
| --- | --- | --- |
| 1 | Single press | Enables/Disables logging (Only for QMAX Internal use) |
| 2 | Long press for 3s – 8s | Enters/Exits the Gateway into/from teaching mode |
| 3 | Long press for >= 9s | Restarts the Gateway |

# Node Button Press Logics

Table 9.1 Node Button Press Logics

| Sl. No | Parameter | Description |
| --- | --- | --- |
| 1 | Single press | Enables/Disables logging (Only for QMAX Internal use) |
| 2 | Long press for 3s – 8s | Enters/Exits the Node into/from teaching mode |
| 3 | Long press for >= 9s | Restarts the Node |

# MQTT Communication Packets

The communication between Gateway and Cloud is based on JSON packets. Communication can take place in both directions i.e., from the gateway to the cloud and vice versa. 

Note: If a packet name has “ACK” at its end, then it means, the origin of the packet is from the device’s side. All other packets have origin from the Cloud end.

Table 10.1 MQTT Communication Packets

| Packet Name | Packet ID | Packet Description |
| --- | --- | --- |
| GATEWAY MQTT PACKETS |  |  |
| Gateway Registration Packet | 0 | Adds a Gwy into BLE Mesh network |
| Gateway AC Remote Configuration ACK | 1 | Sent back as ACK to cloud. Contains information about Gateway AC remote configuration |
| Gateway Unregistration Packet | 2 | Removes a Gwy from BLE Mesh network |
| Gateway AC Control Packet | 3 | Contains AC control settings to control AC using Gwy |
| Gateway AC Manual control ACK | 4 | Sent back as ACK to cloud. Contains information about manual control of AC using remote |
| Gateway AC Remote Reconfiguration Packet | 5 | Puts Gwy into AC remote configuration mode |
| Gateway Heartbeat ACK | 6 | Sent back as ACK to cloud. Contains current AC setting and ambient temperature information |
| Gateway Heartbeat Publish Configuration Packet | 7 | Tells the Gwy, how frequently it needs to send the Heartbeat ACK to cloud |
| Gateway Teaching Mode Start Packet | 8 | Puts the Gateway in AC remote teaching mode |
| Gateway Teaching Mode End ACK | 9 | Sent back as ACK to cloud. Tells if the Gwy has exited Teaching mode or not. |
| Gateway Debug Info Packet | 10 | To get information like Firmware details, Registration, Configuration statuses from Gwy (Only for QMAX Internal use) |
| NODE MQTT PACKETS |  |  |
| Node Provisioning Packet | 100 | Adds a Node into BLE Mesh network |
| Node AC Remote Configuration ACK | 101 | Sent back as ACK to cloud. Contains information about Node AC remote configuration |
| Node Unprovisioning Packet | 102 | Removes a Node from BLE Mesh network |
| Node AC control Packet | 103 | Contains AC control settings to control AC using Node |
| Node AC Manual Control ACK | 104 | Sent back as ACK to cloud. Contains information about manual control of AC using remote |
| Node AC Remote Reconfiguration Packet | 105 | Puts Node into Configuration mode |
| Node Heartbeat ACK | 106 | Sent back as ACK to cloud. Contains ambient temperature value measured by the on-board Temperature sensor on Node |
| Node Heartbeat Publish Configuration Packet | 107 | Tells the Node, how frequently it needs to send the Heartbeat ACK to Gwy |
| Node Teaching Mode Start Packet | 108 | Puts a Node into AC Remote teaching mode. |
| Node Teaching Mode End ACK | 109 | Sent back as ACK to cloud. Tells if the Node has exited Teaching mode or not. |
| Node Debug Info Packet | 110 | To get information like Firmware details, Registration, Configuration statuses from Node (Only for QMAX Internal use) |

## Error codes

Below table contains all the error codes used across the application.

Table 10.2 Error Codes

| Error code | Error code name |
| --- | --- |
| -1 | FAILURE |
| 0 | SUCCESS |
| 1 | JSON_PACKET_ID_NOT_FOUND |
| 2 | JSON_PACKET_ID_UNKNOWN |
| 3 | MSG_SEQ_NO_NOT_FOUND |
| 4 | GWY_SER_NO_NOT_FOUND |
| 5 | GWY_SER_NO_INVALID |
| 6 | LOCATION_NOT_FOUND |
| 7 | LOCATION_EXCEEDING_RANGE |
| 8 | NODE_COMM_TIMEOUT |
| 9 | GWY_NOT_REG |
| 10 | GWY_ALREADY_REG |
| 11 | MAC_ID_NOT_FOUND |
| 12 | MAC_ID_CONTAINS_INVALID_CHARS_OR_INVALID_FORMAT |
| 13 | INVALID_MAC_ID_LENGTH |
| 14 | NODE_SER_NO_NOT_FOUND |
| 15 | ELEMENT_ADDR_NOT_FOUND |
| 16 | GWY_NOT_CONFIGURED_WITH_AC_REMOTE |
| 17 | NODE_NOT_CONFIGURED_WITH_AC_REMOTE |
| 18 | POWER_NOT_FOUND |
| 19 | POWER_EXCEEDING_RANGE |
| 20 | MODE_NOT_FOUND |
| 21 | MODE_EXCEEDING_RANGE |
| 22 | FAN_SPEED_NOT_FOUND |
| 23 | FANSPEED_EXCEEDING_RANGE |
| 24 | TEMPERATURE_NOT_FOUND |
| 25 | TEMPERATURE_EXCEEDING_RANGE |
| 26 | SWING_H_NOT_FOUND |
| 27 | SWING_H_EXCEEDING_RANGE |
| 28 | SWING_V_NOT_FOUND |
| 29 | SWING_V_EXCEEDING_RANGE |
| 30 | ONTIMER_NOT_FOUND |
| 31 | ONTIMER_EXCEEDING_RANGE |
| 32 | OFFTIMER_NOT_FOUND |
| 33 | OFFTIMER_EXCEEDING_RANGE |
| 34 | LOCKING_NOT_FOUND |
| 35 | LOCKING_EXCEEDING_RANGE |
| 36 | TEMP_LOCK_UP_LIMIT_NOT_FOUND |
| 37 | TEMP_LOCK_LOW_LIMIT_NOT_FOUND |
| 38 | TEMP_LOCK_UP_LIMIT_EXCEEDS_ABS_TEMP_UP_LIMIT |
| 39 | TEMP_LOCK_LOW_LIMIT_EXCEEDS_ABS_TEMP_LOW_LIMIT |
| 40 | ILLOGICAL_LOCKING_TEMP_LIMIT |
| 41 | PUBLISH_PERIOD_NOT_FOUND |
| 42 | PUBLISH_PERIOD_EXCEEDS_RANGE |
| 43 | MSG_SEQ_NO_EXCEEDING_RANGE |
| 44 | NODE_SER_NO_INVALID |
| 45 | RESET_DEVICE_NOT_FOUND |
| 46 | LOGGING_FLAG_NOT_FOUND |
| 47 | AC_REMOTE_UNSUPPORTED |
| 48 | JSON_PACKET_INVALID |
| 999 | FORBIDDEN_OPERATION |
| 9999 | UNKNOWN ERROR CODE |

### FAILURE

This error code is generated when the desired operation fails to perform. Although currently not being used in any ACKs.

### SUCCESS

This error code is generated when the desired operation is successful.

### JSON_PACKET_ID_NOT_FOUND

When an MQTT packet is received by the Gateway without a JsonPacketId parameter in it, the following JSON gets sent as ACK:

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “ErrorCode” | : | 1 | // (Number) |
| } |  |  |  |  |

### JSON_PACKET_ID_UNKNOWN

When an MQTT packet is sent with a JsonPacketId that’s not in the Table 10.1, then the following JSON gets sent as ACK:

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “ErrorCode” | : | 2 | // (Number) |
| } |  |  |  |  |

### MSG_SEQ_NO_NOT_FOUND

This error code is generated when any MQTT packet is received by the Gateway without a MsgSeqNo parameter in it.

### GWY_SER_NO_NOT_FOUND

This error code is generated when any MQTT packet is received by the Gateway without a GwySerNo parameter in it.

### GWY_SER_NO_INVALID

This error code is generated when any MQTT packet is received by the Gateway with a GwySerNo parameter that is either of invalid format or doesn’t match with the device’s GwySerNo.

### LOCATION_NOT_FOUND

This error code is generated when any MQTT packet is received by the Gateway without a Location parameter in it.

### LOCATION_EXCEEDING_RANGE

This error code is generated when any MQTT packet is sent with a Location parameter, whose length value exceeds the range mentioned in the packet information.

### NODE_COMM_TIMEOUT

This error code is generated when any Node related MQTT packet is sent from Cloud to Gwy and no ACK from the intended Node was received by the Gwy within 10s from when the packet was sent from Gwy to Node.

### GWY_NOT_REG

This error code is generated when Gwy receives any of the following packets when it remains unregistered:

- Gateway Unregistration Packet

- Gateway AC Control Packet

- Gateway AC Remote Reconfiguration Packet

- Gateway Heartbeat Publish Configuration Packet

- Gateway Teaching Mode Start Packet

- Node Provisioning Packet

- Node Unprovisioning Packet

- Node AC Control Packet

- Node AC Remote Reconfiguration Packet

- Node Heartbeat Publish Configuration Packet

- Node Teaching Mode Start Packet

- Node Debug Info Packet

### GWY_ALREADY_REG

This error code is generated when Gwy receives Gateway Registration Packet when it is already a registered device.

### MAC_ID_NOT_FOUND

This error code is generated when any MQTT packet is received by the Gateway without a MacId parameter in it.

### MAC_ID_CONTAINS_INVALID_CHARS_OR_INVALID_FORMAT

This error code is generated when any MQTT packet is sent with a MacId that consists of any of the following:

- Contains Invalid characters. Valid Mac Id will contain only [0-9], [a-f], [A-F].

- Contains Invalid Format. Valid Mac Id format in our case is 6 sets of 2-characters separated by a colon between them. Like this: AB:56:7F:88:9E:62

### INVALID_MAC_ID_LENGTH

This error code is generated when any MQTT packet is sent with a MacId that’s of Invalid Length. A Valid MacId in our case should be 17 characters in length. (including Colon)

### NODE_SER_NO_NOT_FOUND

This error code is generated when any Node Related MQTT packet is received by the Gateway without a NodeSerNo parameter in it.

### ELEMENT_ADDR_NOT_FOUND

This error code is generated when any Node Related MQTT packet (Except Node Provisioning Packet) is sent without an ElementAddr parameter in it.

### GWY_NOT_CONFIGURED_WITH_AC_REMOTE

This error code is generated when Gwy receives any of the following packets when it remains Unconfigured:

- Gateway AC Control Packet

- Gateway AC Remote Reconfiguration Packet

### NODE_NOT_CONFIGURED_WITH_AC_REMOTE

This error code is generated when Gwy receives any of the following packets when the intended Node remains Unconfigured:

- Node AC Control Packet

- Node AC Remote Reconfiguration Packet

### POWER_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a Power parameter in it.

### POWER_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a Power parameter, whose value is neither 1 nor 0.

### MODE_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a Mode parameter in it.

### MODE_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a Mode parameter, whose value is neither of the following:

- Cool

- Hot

- Dry

- Auto

- Fan

Please make note, the above is CASE SENSITIVE.

### FAN_SPEED_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a FanSpeed parameter in it

### FANSPEED_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a FanSpeed parameter, whose value is not between 0 – 5.

### TEMPERATURE_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a Temperature parameter in it.

### TEMPERATURE_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a Temperature parameter, whose value is not between 18 – 32.

### SWING_H_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a SwingH parameter in it.

### SWING_H_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a SwingH parameter, whose value is neither 1 nor 0.

### SWING_V_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a SwingV parameter in it.

### SWING_V_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a SwingV parameter, whose value is neither 1 nor 0.

### ONTIMER_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without an OnTimer parameter in it.

### ONTIMER_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with an OnTimer, whose value is not between 0 – 12.

### OFFTIMER_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without an OffTimer parameter in it.

### OFFTIMER_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with an OffTimer parameter, whose value is not between 0 – 12.

### LOCKING_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a Locking parameter in it.

### LOCKING_EXCEEDING_RANGE

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a Locking parameter, whose value is neither 1 nor 0.

### TEMP_LOCK_UP_LIMIT_NOT_FOUND

This error code is generated when a Gateway AC Control Packet  or Node AC control packet is received by the Gateway without a TempLockUpLimit parameter in it.

### TEMP_LOCK_LOW_LIMIT_NOT_FOUND

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway without a TempLockLowLimit parameter in it.

### TEMP_LOCK_UP_LIMIT_EXCEEDS_ABS_TEMP_UP_LIMIT

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a TempLockUpLimit parameter, whose value is greater than 32.

### TEMP_LOCK_LOW_LIMIT_EXCEEDS_ABS_TEMP_LOW_LIMIT

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a TempLockLowLimit parameter, whose value is less than 18.

### ILLOGICAL_LOCKING_TEMP_LIMIT

This error code is generated when a Gateway AC Control Packet or Node AC control packet is received by the Gateway with a TempLockLowLimit parameter, whose value is higher than TempLockUpLimit parameter’s value.

### PUBLISH_PERIOD_NOT_FOUND

This error code is generated when a Gateway Heartbeat Publish Configuration Packet or Node Heartbeat Publish Configuration packet is received by the Gateway without a PublishPeriodSec parameter in it.

### PUBLISH_PERIOD_EXCEEDS_RANGE

This error code is generated when a Gateway Heartbeat Publish Configuration Packet or Node Heartbeat Publish Configuration packet is received by the Gateway with a PublishPeriodSec parameter, whose value is not between 10 – 255.

### MSG_SEQ_NO_EXCEEDING_RANGE

This error code is generated when any MQTT packet is received by the Gateway with a MsgSeqNo parameter, whose value is not between 0 – 65535.

### NODE_SER_NO_INVALID

This error code is generated when any MQTT packet is received by the Gateway with a NodeSerNo parameter that is either of invalid format or doesn’t match with the intended Node’s NodeSerNo. Due to technical difficulties, this will not be applicable for Node Provisioning packet

### RESET_DEVICE_NOT_FOUND

This error code is generated when Gwy Debug Info or Node Debug Info packet is received by the Gwy without ResetDevice parameter in it.

### LOGGING_FLAG_NOT_FOUND

This error code is generated when Gwy Debug Info or Node Debug Info packet is received by the Gwy without Logging parameter in it.

### FORBIDDEN_OPERATION

Reserved for future use.

### UNKNOWN ERROR CODE

Reserved for future use

### AC_REMOTE_UNSUPPORTED

This error code is generated when an unsupported AC remote is used for AC Remote configuration process.

### JSON_PACKET_INVALID

This error code is generated when any incorrectly formatted JSON message is received from the cloud. If the format is incorrect, it’s not possible to parse that JSON message any further.

## Common JSON parameters

The following parameters are used across most of the MQTT communication packets. Below table provides information about these parameters and the purpose behind them.

Table 10.3 Common JSON Parameter description

| Sl. No | Parameter | Description |
| --- | --- | --- |
| 1 | JsonPacketId | This is used for identification on both cloud side and Gwy side to know which packet is being received |
| 2 | MsgSeqNo | The message sequence number is used to identify which ACK corresponds to which command. |
| 3 | GwySerNo | Serial Number to identify a particular Gwy. |
| 4 | Error Code | Included on all ACK packets. Let’s the cloud know if the operation has succeeded or needs attention. |
| 5 | Location | This is used to identify the physical location of where the device was installed. User sets this information upon Registration / Provision process. |
| 6 | NodeSerNo | Serial Number to identify a particular Node. |
| 7 | ElementAddr | A Number used for communication to Node inside the BLE network. |

## Gateway Registration Packet

Description: This packet is used to add a Gwy device into the BLE Mesh network.

LED Indication: When in Unregistered state, Gateway blinks RED + BLUE.

Gateway Registration JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 0 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “Location” | : | “1st Floor” | // (String) | [30 chars long] |
| } |  |  |  |  |  |

Process flow diagram:

Figure 10.1 Gateway Registration process flow

## Gateway Registration ACK

Description: This ACK is used by cloud to know whether the Gwy registration was successful or not. Gateway sends back this packet after receiving Gateway Registration packet.

Gateway Registration ACK JSON (Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 0 | // (Number) |  |
|  | “JsonAckName” | : | “Gwy Registration Ack” | // (String) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “Location” | : | “1st Floor” | // (String) | [30 chars long] |
|  | “ErrorCode” | : | 0 | // (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow diagram:

Figure 10.2 Gateway Registration process flow

## Gateway AC Remote Configuration ACK

Description: This is an ACK sent by Gwy to Cloud when the Gwy is successfully configured with an AC remote. When Gwy is in AC Remote Configuration mode (Slow Blinking BLUE light), and AC remote is pressed in-front of the IR receiver in the Gwy, it will get configured to act as that AC remote from there on and send this ACK to the cloud.

LED Indication: If not configured, Gateway will glow Solid BLUE LED, meaning the Gateway is in AC Remote configuration mode.

Gateway AC Remote Configuration ACK JSON (Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 1 | // (Number) |  |
|  | “JsonAckName” | : | “Gwy AC Remote Configuration Ack” | // (String) |  |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “ErrorCode” | : | 0 | // (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.3 Gateway Configuration process flow

## Gateway Unregistration Packet

Description: This packet is used to remove a Gwy device from the BLE Mesh network.

Note: When a Gateway is Unregistered, it will reset itself to factory settings. But it will still retain its current mesh network information.

Gateway Unregistration JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 2 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “Location” | : | “1st Floor” | // (String) | [30 chars long] |
| } |  |  |  |  |  |

Process flow:

Figure 10.4 Gateway Unregistration Process flow

## Gateway Unregistration ACK

Description: This ACK is used by cloud to know whether the Gwy Unregistration was successful or not. This packet will be sent by Gateway after receiving Gateway Unregistration packet.

Gateway Unregisration ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 2 | // (Number) |
|  | “JsonAckName” | : | “Gwy Unregistration Ack” | // String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “Location” | : | “1st Floor” | // (String) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.5 Gateway Unregistration Process Flow

## Gateway AC Control Packet

Description: This packet is used to control an AC using the Gwy. The below table describes the AC control parameters that go with the Gateway AC Control Packet.

Table 10.4 AC Control Parameters

| Sl. No | Parameter | Description |
| --- | --- | --- |
| 1 | Power | 1 = ON, 0 = OFF |
| 2 | Mode | The following modes are supported: [Cool, Dry, Hot, Fan, auto]. |
| 3 | FanSpeed | [1 - 5] levels |
| 3 | Temperature | Sets the AC temperature. Must be between 18 and 32. |
| 4 | SwingH | 1 = ON, 0 = OFF |
| 5 | SwingV | 1 = ON, 0 = OFF |
| 6 | OnTimer | Sets the number of hours past midnight after which AC needs to be turned on automatically. [0 – 12] |
| 7 | OffTimer | Sets the number of hours past midnight after which AC needs to be turned off automatically [0 – 12] |
| 8 | Locking | 1 = ON, 0 = OFF. When ON, if AC was controlled manually, then Gateway Manual AC Control ACK will be sent to Cloud. Also, when ON, the AC will be maintained between the TempLockUpLimit and TempLockLowLimit. If by manual control, AC temp was set exceeding the above limits, then Gwy will emit IR Signal to control the AC and bring it back to whatever setting was set by Gwy lastly. |
| 9 | TempLockUpLimit | Specifies upper temperature locking limit. This parameter will have no effect application wise, when Locking is not enabled. Must have values within the absolute Temperature limits: 18 - 32. |
| 10 | TempLockLowLimit | Specifies Lower temperature locking limit. When Locking is not enabled, this parameter will have no effect application wise. Must have values within the absolute Temperature limits: 18 - 32. |

Note: Gateway Manual AC control ACK is applicable only when devices have not undergone the Teaching Mode Process. For Devices, using the Teaching Mode to control ACs, Gateway Manual AC control ACK will be sent with all the AC parameter values set to 0. Example ACK for both the cases is provided in the Gateway Manual AC control ACK section.

Gateway AC Control JSON packet (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 3 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “Power” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “Mode” | : | “Cool” | // (String) | [“Cool“, “Dry“, “Auto“, “Fan“, “Hot“] |
|  | “FanSpeed” | : | 1 | // (Number) | [1 - 5] |
|  | “Temperature” | : | 25 | // (Number) | [18 - 32] |
|  | “SwingH” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “SwingV” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “OnTimer” | : | 0 | // (Number) | [0 - 12] |
|  | “OffTimer” | : | 0 | // (Number) | [0 - 12] |
|  | “Locking” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “TempLockUpLimit” | : | 26 | // (Number) | [18 - 32] |
|  | “TempLockLowLimit” | : | 20 | // (Number) | [18 - 32] |
| } |  |  |  |  |  |

Process flow:

Figure 10.6 Gateway AC Control Process flow

## Gateway AC Control ACK

Description: This ACK is used by cloud to know whether the Gwy AC Control was successful or not.

Gateway AC Control ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 3 | // (Number) |
|  | “JsonAckName” | : | “Gwy AC Control Ack” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 1 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
|  | “SwingH” | : | 1 | // (Number) |
|  | “SwingV” | : | 1 | // (Number) |
|  | “OnTimer” | : | 0 | // (Number) |
|  | “OffTimer” | : | 0 | // (Number) |
|  | “Locking” | : | 1 | // (Number) |
|  | “TempLockUpLimit” | : | 26 | // (Number) |
|  | “TempLockLowLimit” | : | 20 | // (Number) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.7 Gateway AC Control Process flow

## Gateway AC Manual control ACK

Description: If Locking was enabled, this ACK is used by cloud to know whether the AC was controlled manually using the AC remote. If someone controls the AC manually using the remote, then the IR receiver on-board will capture this IR signal and decode it. If it’s a signal from the same remote with which the Gateway was configured during the AC Remote configuration process, then it will store the manual control information for sending it to the cloud. Also, it performs logical operation based on the temperature that was set using the remote. If it exceeds the Locking temperature limits set from the AC Control packet, then it will bring the AC’s temperature back to permissible limits.

Note: The Gateway Manual Control ACK will work only for Devices that are not using Teaching Mode to control AC.

Gateway AC Manual Control ACK JSON (Gateway to Cloud) (Non-Teaching Mode case):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 4 | // (Number) |
|  | “JsonAckName” | : | “Gwy AC Manual Control ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 1 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
| } |  |  |  |  |

Gateway AC Manual Control ACK JSON (Gateway to Cloud) (Teaching Mode case):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 4 | // (Number) |
|  | “JsonAckName” | : | “Gwy AC Manual Control ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “Power” | : | 0 | // (Number) |
|  | “Mode” | : | “” | // (String) |
|  | “FanSpeed” | : | 0 | // (Number) |
|  | “Temperature” | : | 0 | // (Number) |
| } |  |  |  |  |

Process flow:

Figure 10.8 Gateway AC manual control Process flow

## Gateway AC Remote Reconfiguration Packet

Description: This packet is used to put the Gwy in AC remote configuration mode.

Gateway AC Remote Reconfiguration JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 5 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
| } |  |  |  |  |  |

Process flow:

Figure 10.9 Gateway AC Remote Reconfiguration Process Flow

## Gateway AC Remote Reconfiguration ACK

Description: This ACK is used by cloud to know if the Gwy has entered AC Remote reconfiguration mode after receiving the Gateway AC Remote Reconfiguration packet.

Led Indication: Gwy will glow solid BLUE to indicate it has entered Gwy AC Remote configuration mode.

Gateway AC Remote Reconfiguration ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 5 | // (Number) |
|  | “JsonAckName” | : | “Gwy AC Remote Reconfiguration Ack” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.10 Gateway AC Remote Reconfiguration Process flow

## Gateway Heartbeat ACK

Description: This ACK is used by cloud to know the Ambient temperature of the environment in which the Gwy is operating as well as the current AC control settings. This ACK is sent periodically to the cloud. This frequency can be configured in the Gateway Heartbeat Publish Configuration Packet.

Gateway Heartbeat ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 6 | // (Number) |
|  | “JsonAckName” | : | “Gwy Heartbeat ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 3 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
|  | “AmbientTemperature” | : | 25 | // (Number) (Measured from sensor) |
|  | “SwingH” | : | 1 | // (Number) |
|  | “SwingV” | : | 0 | // (Number) |
|  | “OnTimer” | : | 5 | // (Number) |
|  | “OffTimer” | : | 12 | // (Number) |
|  | “Locking” | : | 0 | // (Number) |
|  | “TempLockUpLimit” | : | 27 | // (Number) |
|  | “TempLockLowLimit” | : | 20 | // (Number) |
| } |  |  |  |  |

Process flow:

Figure 10.11 Gateway Heartbeat ACK Process flow

## Gateway Heartbeat Publish Configuration Packet

Description: This packet is used to tell the Gwy, how frequently it needs to send the Heartbeat ACK to cloud. The frequency is configurable in seconds. The default Value is 10s.

Gateway Heartbeat Publish Configuration JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 7 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00000 - 99999] |
|  | “PublishPeriodSec” | : | 20 | // (Number) (in Sec) | [300 - 65535] |
| } |  |  |  |  |  |

Process flow:

Figure 10.12 Gateway Heartbeat Publish Configuration Process flow

## Gateway Heartbeat Publish Configuration ACK

Description: This ACK is used by cloud to know whether the Gateway Heartbeat Publish configuration operation was successful or not.

Gateway Heartbeat Publish Configuration ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 7 | // (Number) |
|  | “JsonAckName” | : | “Gwy Heartbeat Publish Configuration ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “PublishPeriodSec” | : | 20 | // (Number) (in Sec) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.13 Gateway Heartbeat Publish Configuration ACK Process flow

## Gateway Teaching Mode Start Packet

Description: This packet is used to make a Gateway enter Teaching mode.

Led Indication: Gwy will glow Blinking BLUE to indicate it has entered Teaching mode.

Gateway Teaching Mode Start JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 8 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
| } |  |  |  |  |  |

Process flow:

Figure 10.14 Gateway Teaching Mode Start Flow

Steps to Perform Teaching Mode:

- Make sure Gateway is registered / Make sure Node is provisioned.

- IMPORTANT NOTE (before making the device enter Teaching mode): Take the AC remote and bring down the temperature to 19 and power ON the AC. So that, when you press the power button again, it should send the POWER OFF signal. This is very important in-order to successfully register the commands to EEPROM on-board and complete the teaching process. The sequence of IR signals for teaching mode is as follows:

  - POWER OFF | TEMP 19

  - POWER ON | TEMP 19

  - POWER ON | TEMP 20

  - POWER ON | TEMP 21

  - POWER ON | TEMP 22

  - POWER ON | TEMP 23

  - POWER ON | TEMP 24

  - POWER ON | TEMP 25

  - POWER ON | TEMP 26

  - POWER ON | TEMP 27

  - POWER ON | TEMP 28

- Long press the button for 3s – 8s or use the Gateway Teaching Mode Start Packet to put the device into Teaching Mode. The LED will start Fast Blinking BLUE (once every 200ms) to indicate it has entered teaching mode.

- Press the POWER ON button on the AC remote with remote pointing towards to IR receiver on-board for successful reception and prevent false recording.

- The device will read the IR signal from the remote and store it in flash. While this process is happening, the light will turn off momentarily to indicate that no other buttons on AC remote must be pressed to let the process go on without disturbance. Also, make sure you are doing this process in an IR disturbance free environment. Most smartphones these days use IR emitters, so it's one thing that I experienced during my development.

- Once the LED starts blinking BLUE again, it's time to record the next IR signal. The next signal is POWER ON | TEMP 19.

- Similarly, go one-by-one all the way up to POWER ON | TEMP 28. If everything was done right, then, when the last IR signal was sent, the LED will change to SOLID GREEN to indicate successful completion of the Teaching Mode process

Steps to Exit Teaching Mode:

- When already in Teaching Mode, if the button is long pressed for 3s-8s, device will exit teaching mode.

## Gateway Teaching Mode Start ACK

Description: This ACK is used by the cloud to know if the Gwy has successfully entered the teaching mode or not after receiving Gateway Teaching Mode Start Packet.

Led Indication: Gwy will glow Blinking BLUE to indicate it has entered Teaching Mode.

Gateway Teaching Mode Start ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 8 | // (Number) |
|  | “JsonAckName” | : | “Gwy Teaching Mode Start ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.15 Gateway Teaching Mode Start ACK Flow

## Gateway Teaching Mode End ACK

Description: This ACK is used by the cloud to know if the Gwy has exited Teaching mode.

Led Indication: On completion of Teaching mode, the Gwy will glow Solid GREEN.

Gateway Teaching Mode End ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 9 | // (Number) |
|  | “JsonAckName” | : | “Gwy Teaching Mode End ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
| } |  |  |  |  |

Process flow:

Figure 10.16 Gateway Teaching Mode End ACK Flow

## Gateway Debug Info Packet (Only for QMAX Internal use)

Description: This packet is sent to retrieve information from a Gwy device regarding the firmware version it’s currently running, Registration-Configuration statuses, Queue Count, etc., This packet also contains a parameter called logging, which can be used to enable/disable logging during runtime. Logging can also be enabled/disable by Single pressing the button on the device.

Gateway Debug Info Packet JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 10 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “Logging” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “ResetDevice” | : | 1 | // (Number) | [0 = False, 1 = True] |
| } |  |  |  |  |  |

Process flow:

Figure 10.17 Gateway Debug Info Process Flow

## Gateway Debug Info ACK (Only for QMAX Internal use)

Description: This ACK is used by Cloud to utilize Debugging information from a Gwy.

Gateway Debug Info Packet ACK JSON (Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 10 | // (Number) |
|  | “JsonAckName” | : | “Gwy Debug Info ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “FirmwareVersion” | : | “1.1” | // (String) (Maj.Min) |
|  | “Registered” | : | 1 | // (Number) |
|  | “Protocol” | : | DAIKIN216 | // (String) |
|  | “PublishPeriodSec” | : | 300 | // (Number) |
|  | “PubMsgQueueCount” | : | 0 | // (Number) |
|  | “ProvQueueCount” | : | 3 | // (Number) |
|  | “UnProvQueueCount” | : | 4 | // (Number) |
|  | “ACControlQueueCount” | : | 2 | // (Number) |
|  | “ReconfQueueCount” | : | 5 | // (Number) |
|  | “PubConfQueueCount” | : | 2 | // (Number) |
|  | “TeachingModeQueueCount” | : | 8 | // (Number) |
|  | “DebugInfoQueueCount” | : | 1 | // (Number) |
|  | “DeviceUpTimeHrs” | : | “0.54” | // (String) (in hrs) |
|  | “Logging” | : | 1 | // (Number) |
|  | “ResetDevice” | : | 1 | // (Number) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process Flow:

Figure 10.18 Gateway Debug Info ACK flow

## Node Provision Packet

Description: This packet is used to add a Node device into the BLE Mesh network

Node Provisioning JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 100 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo: | : | “N00001” | // (String) | [00001 - 99999] |
|  | “MacId” | : | “D3:F5:FB:CA:14:25” | // (String) (MacID of the Node) | [17 chars long] |
|  | “Location” | : | “1st Floor” | // (Number) | [30 chars long] |
| } |  |  |  |  |  |

Process flow:

Figure 10.19 Node Provisioning Flow

## Node Provision ACK

Description: This ACK is used by cloud to know whether the Node provisioning was successful or not.

Note: In this ACK, the ElementAddr is a very important field, whose value should be noted for further Node operations. Ony using this ElementAddr, the Gateway will be able to communicate with the Node. This ElementAddr field will be included in all Node related MQTT packets.

Node Provisioning ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 100 | // (Number) |  |
|  | “JsonAckName” | : | “Node Provisioning ACK” | // (String) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) |  |
|  | “GwySerNo” | : | “GWY00001” | // (String) |  |
|  | “NodeSerNo: | : | “N00001” | // (String) |  |
|  | “MacId” | : | “D3:F5:FB:CA:14:25” | // (String) (MacID of the Node) |  |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “Location” | : | “1st Floor” | // (String) |  |
|  | “AppKeyIndex” | : | 1 | // (Number) |  |
|  | “AppKey” | : | 29 | // (Number) |  |
|  | “NetKeyIndex” | : | 5 | // (Number) |  |
|  | “NetKey” | : | 37 | // (Number) |  |
|  | “ErrorCode” | : | 0 | // (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.20 Node Provisioning ACK Flow

## Node AC Remote Configuration ACK

Description: This is an ACK sent by Node to Gwy to Cloud when the Node is successfully configured with an AC remote. When the Node is in AC Remote Configuration mode and AC remote is pressed in-front of the IR receiver in the Node, it will get configured to act as that AC remote from there on.

LED Indication: If the Node has not been AC Remote configured, the Node will glow Solid BLUE LED.

Node Configuration ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 101 | // (Number) |  |
|  | “JsonAckName” | : | “Node AC Remote Configuration ACK” | // (String) |  |
|  | “GwySerNo” | : | “GWY00001” | // (String) |  |
|  | “NodeSerNo: | : | “N00001” | // (String) |  |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “ErrorCode” | : | 0 | // (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.21 Node Configuration Process flow

## Node Unprovision Packet

Description: This packet is used to remove a Node device from the BLE Mesh network.

Led Indication: After successful Unprovisioning, Node will start Blinking RED + BLUE.

Node Unprovisioning JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 102 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) |  |
|  | “GwySerNo” | : | “GWY00001” | // (String) |  |
|  | “NodeSerNo: | : | “N00001” | // (String) |  |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “Location” | : | “1st Floor” | // (String) | [30 chars long] |
| } |  |  |  |  |  |

Process flow:

Figure 10.22 Node Unprovsioning Process flow

## Node Unprovision ACK

Description: This packet is used to add a Node device into the BLE Mesh network

Node Unprovisioning ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 102 | // (Number) |
|  | “JsonAckName” | : | “Node Unprovisioning ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo: | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “Location” | : | “1st Floor” | // (String) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.23 Node Unprovisioning Process flow

## Node AC Control Packet

Description: This packet is used to control an AC using the Node. The below table describes the AC control parameters that go with the Node AC Control Packet.

Table 10.5 AC Control Parameters

| Sl. No | Parameter | Description |
| --- | --- | --- |
| 1 | Power | 1 = ON, 0 = OFF |
| 2 | Mode | The following modes are supported: [Cool, Dry, Hot, Fan, auto]. |
| 3 | FanSpeed | [1 - 5] levels |
| 3 | Temperature | Sets the AC temperature. Must be between 18 and 32. |
| 4 | SwingH | 1 = ON, 0 = OFF |
| 5 | SwingV | 1 = ON, 0 = OFF |
| 6 | OnTimer | Sets the number of hours past midnight after which AC needs to be turned on automatically. [0 – 12] |
| 7 | OffTimer | Sets the number of hours past midnight after which AC needs to be turned off automatically [0 – 12] |
| 8 | Locking | 1 = ON, 0 = OFF. When ON, if AC was controlled manually, then Node Manual AC Control ACK will be sent to Cloud. Also, when ON, the AC will be maintained between the TempLockUpLimit and TempLockLowLimit. If by manual control, AC temp was set exceeding the above limits, then Node will emit IR Signal to control the AC and bring it back to whatever setting was set by Node lastly. |
| 9 | TempLockUpLimit | Specifies upper temperature locking limit. This parameter will have no effect application wise, when Locking is not enabled. Must have values within the absolute Temperature limits: 18 - 32. |
| 10 | TempLockLowLimit | Specifies Lower temperature locking limit. When Locking is not enabled, this parameter will have no effect application wise. Must have values within the absolute Temperature limits: 18 - 32. |

Note: Node Manual AC control ACK is applicable only when devices have not undergone the Teaching Mode Process. For Devices, using the Teaching Mode to control ACs, Node Manual AC control ACK will be sent with all the AC parameter values set to 0. Example ACK for both the cases is provided in the Node Manual AC control ACK section.

Node AC Control JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 103 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 - 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “Power” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “Mode” | : | “Cool” | // (String) | [“Cool“, “Dry“, “Auto“, “Fan“, “Hot“] |
|  | “FanSpeed” | : | 1 | // (Number) | [1 - 5] levels |
|  | “Temperature” | : | 25 | // (Number) | [18 - 32] |
|  | “SwingH” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “SwingV” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “OnTimer” | : | 0 | // (Number) | [0 - 12] |
|  | “OffTimer” | : | 0 | // (Number) | [0 - 12] |
|  | “Locking” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “TempLockUpLimit” | : | 26 | // (Number) | [18 - 32] |
|  | “TempLockLowLimit” | : | 20 | // (Number) | [18 - 32] |
| } |  |  |  |  |  |

Process flow:

Figure 10.24 Node AC Control Process flow

## Node AC Control ACK

Description: This is used by cloud to know if the AC Control was successful or not.

Node AC Control ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 103 | // (Number) |
|  | “JsonAckName” | : | “Node AC Control ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo” | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 1 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
|  | “SwingH” | : | 1 | // (Number) |
|  | “SwingV” | : | 1 | // (Number) |
|  | “OnTimer” | : | 0 | // (Number) |
|  | “OffTimer” | : | 0 | // (Number) |
|  | “Locking” | : | 1 | // (Number) |
|  | “TempLockUpLimit” | : | 26 | // (Number) |
|  | “TempLockLowLimit” | : | 20 | // (Number) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.25 Node AC Control Process flow

## Node AC Manual control ACK

Description: If Locking was enabled, this ACK is used by cloud to know whether the AC was controlled manually using the AC remote. If someone controls the AC manually using the remote, then the IR receiver on-board will capture this IR signal and decode it. If it’s a signal from the same remote with which the Node was configured during the AC Remote configuration process, then it will store the manual control information for sending it to the cloud. Also, it performs logical operation based on the temperature that was set using the remote. If it exceeds the Locking temperature limits set from the Node AC Control packet, then it will bring the AC’s temperature back to permissible limits.

Note: The Node Manual Control ACK will work only for Devices that are not using Teaching Mode to control AC.

Node AC Manual Control ACK JSON (Node to Gateway to Cloud) (Non-Teaching Mode case):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 104 | // (Number) |
|  | “JsonAckName” | : | “Node AC Manual Control ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo: | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 1 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
| } |  |  |  |  |

Node AC Manual Control ACK JSON (Node to Gateway to Cloud) (Teaching Mode case):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 104 | // (Number) |
|  | “JsonAckName” | : | “Node Manual AC Control ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo: | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “Power” | : | 0 | // (Number) |
|  | “Mode” | : | “” | // (String) |
|  | “FanSpeed” | : | 0 | // (Number) |
|  | “Temperature” | : | 0 | // (Number) |
| } |  |  |  |  |

Process flow:

Figure 10.26 Node AC Locking Process flow

## Node AC Remote Reconfiguration Packet

Description: This packet is used to reset the MQTT parameters saved in the Gateway. This puts the Node into AC Remote Configuration mode.

LED Indication: Node will glow Solid BLUE to indicate the Node has entered AC Remote Configuration mode

Node AC Remote Reconfiguration JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 105 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00000 - 99999] |
|  | “NodeSerNo: | : | “N00001” | // (String) |  |
|  | “ElementAddr” | : | 23 | // (Number) |  |
| } |  |  |  |  |  |

Process flow:

Figure 10.27 Node Reconfiguration Process flow

## Node AC Remote Reconfiguration ACK

Description: This ACK is used by cloud to know whether the node successfully entered AC Remote configuration mode or not, after receiving the Node AC Remote Reconfiguration Packet.

LED Indication: Node will glow Solid BLUE to indicate the Node has entered AC Remote Configuration mode

Node Reconfiguration ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 105 | // (Number) |  |
|  | “JsonAckName” | : | “Node AC Remote Reconfiguration ACK” | // (String) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) |  |
|  | “GwySerNo” | : | “GWY00001” | // (String) |  |
|  | “NodeSerNo: | : | “N00001” | // (String) |  |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “ErrorCode” | : | 0 | // (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.28 Node Reconfiguration Process flow

## Node Heartbeat ACK

Description: This ACK is used by cloud to know the ambient temperature of the environment in which the Node is operating. This ACK is sent periodically to the Gwy to Cloud. This frequency can be configured in the Node Heartbeat Publish Configuration Packet

Node Heartbeat ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 106 | // (Number) |
|  | “JsonAckName” | : | “Node Heartbeat ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo: | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “Power” | : | 1 | // (Number) |
|  | “Mode” | : | “Cool” | // (String) |
|  | “FanSpeed” | : | 3 | // (Number) |
|  | “Temperature” | : | 25 | // (Number) |
|  | “AmbientTemperature” | : | 23 | // (Number) |
|  | “SwingH” | : | 1 | // (Number) |
|  | “SwingV” | : | 0 | // (Number) |
|  | “OnTimer” | : | 5 | // (Number) |
|  | “OffTimer” | : | 12 | // (Number) |
|  | “Locking” | : | 0 | // (Number) |
|  | “TempLockUpLimit” | : | 27 | // (Number) |
|  | “TempLockLowLimit” | : | 20 | // (Number) |
| } |  |  |  |  |

Process flow:

Figure 10.29 Node Heartbeat Process flow

## Node Heartbeat Publish Configuration Packet

Description: This packet is used to tell the Node, how frequently It needs to send Heartbeat ACK to Gwy, so that Gwy can send it to Cloud. This frequency is configurable in seconds. The default value is 10s.

Node Temperature Publish Configuration JSON (Cloud to Gateway):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 107 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “PublishPeriodSec” | : | 15 | // (Number) (in sec) | [300 - 65535] |
| } |  |  |  |  |  |

Process flow:

Figure 10.30 Node Publish Configuration process flow

## Node Heartbeat Publish Configuration ACK

Description: This ACK is used by cloud to know if the Node publish configuration was successful or not.

Node Publish Configuration ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 107 | (Number) |  |
|  | “JsonAckName” | : | “Node Heartbeat Publish Configuration ACK” | (String) |  |
|  | “MsgSeqNo” | : | 12 | (Number) |  |
|  | “GwySerNo” | : | “GWY00001” | (String) |  |
|  | “NodeSerNo” | : | “N00001” | (String) |  |
|  | “ElementAddr” | : | 23 | (Number) | [2 - 65535] |
|  | “PublishPeriodSec” | : | 15 | (Number) |  |
|  | “ErrorCode” | : | 0 | (Number) |  |
| } |  |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.31 Node Publish Configuration process flow

## Node Teaching Mode Start Packet

Description: This packet is used to make a Node enter Teaching mode.

Led Indication: Node will glow Blinking BLUE to indicate it has entered Teaching mode.

Node Teaching Mode Start JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 108 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
| } |  |  |  |  |  |

Process flow:

Figure 10.32 Node Teaching Mode Start Flow

Steps to Perform Teaching Mode:

- Make sure Gateway is registered / Make sure Node is provisioned.

- IMPORTANT NOTE (before making the device enter Teaching mode): Take the AC remote and bring down the temperature to 19 and power ON the AC. So that, when you press the power button again, it should send the POWER OFF signal. This is very important in-order to successfully register the commands to EEPROM on-board and complete the teaching process. The sequence of IR signals for teaching mode is as follows:

  - POWER OFF | TEMP 19

  - POWER ON | TEMP 19

  - POWER ON | TEMP 20

  - POWER ON | TEMP 21

  - POWER ON | TEMP 22

  - POWER ON | TEMP 23

  - POWER ON | TEMP 24

  - POWER ON | TEMP 25

  - POWER ON | TEMP 26

  - POWER ON | TEMP 27

  - POWER ON | TEMP 28

- Long press the button for 3s – 8s or use the Gateway Teaching Mode Start Packet to put the device into Teaching Mode. The LED will start Fast Blinking BLUE (once every 200ms) to indicate it has entered teaching mode.

- Press the POWER ON button on the AC remote with remote pointing towards to IR receiver on-board for successful reception and prevent false recording.

- The device will read the IR signal from the remote and store it in flash. While this process is happening, the light will turn off momentarily to indicate that no other buttons on AC remote must be pressed to let the process go on without disturbance. Also, make sure you are doing this process in an IR disturbance free environment. Most smartphones these days use IR emitters, so it's one thing that I experienced during my development.

- Once the LED starts blinking BLUE again, it's time to record the next IR signal. The next signal is POWER ON | TEMP 19.

- Similarly, go one-by-one all the way up to POWER ON | TEMP 28. If everything was done right, then, when the last IR signal was sent, the LED will change to SOLID GREEN to indicate successful completion of the Teaching Mode process

Steps to Exit Teaching Mode:

- When already in Teaching Mode, if the button is long pressed for 3s-8s, device will exit teaching mode.

## Node Teaching Mode Start ACK

Description: This ACK is used by the cloud to know if the Node has successfully entered the teaching mode or not after receiving Node Teaching Mode Start Packet.

Led Indication: Node will glow Blinking BLUE to indicate it has entered Teaching mode

Node Teaching Mode start ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 108 | // (Number) |
|  | “JsonAckName” | : | “Node Teaching Mode Start ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo” | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process flow:

Figure 10.33 Node Teaching Mode Start ACK flow

## Node Teaching Mode End ACK

Description: This ACK is used by the cloud to know if the Node has exited teaching mode. Based on Error code, the user will know whether the teaching process was completed fully or not.

LED Indication: After completing the Teaching process, the LED will change to Solid GREEN.

Node Teaching Mode End ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 109 | // (Number) |
|  | “JsonAckName” | : | “Node Teaching Mode End ACK” | // (String) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo” | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
| } |  |  |  |  |

Process flow:

Figure 10.34 Node Teaching Mode End flow

## Node Debug Info Packet (Only for QMAX Internal use)

Description: This packet is sent to retrieve information from a Node device regarding the firmware version it’s currently running, AC Remote configuration status, Device Uptime, etc., This packet also contains a parameter called logging, which can be used to enable/disable logging during runtime. Logging can also be enabled/disable by Single pressing the button on the device.

Node Debug Info Packet JSON (Cloud to Gateway to Node):

| { |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 110 | // (Number) |  |
|  | “MsgSeqNo” | : | 12 | // (Number) | [0 – 65535] |
|  | “GwySerNo” | : | “GWY00001” | // (String) | [00001 - 99999] |
|  | “NodeSerNo” | : | “N00001” | // (String) | [00001 - 99999] |
|  | “ElementAddr” | : | 23 | // (Number) | [2 - 65535] |
|  | “Logging” | : | 1 | // (Number) | [0 = OFF, 1 = ON] |
|  | “ResetDevice” | : | 1 | // (Number) | [0 = False, 1 = True] |
| } |  |  |  |  |  |

Process flow:

Figure 10.35 Node Debug Info Process Flow

## Node Debug Info ACK (Only for QMAX Internal use)

Description: This ACK is used for Debugging purposes.

Node Debug Info Packet ACK JSON (Node to Gateway to Cloud):

| { |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | “JsonPacketId” | : | 110 | // (Number) |
|  | “JsonAckName” | : | “Node Debug Info ACK” | // (String) |
|  | “MsgSeqNo” | : | 12 | // (Number) |
|  | “GwySerNo” | : | “GWY00001” | // (String) |
|  | “NodeSerNo” | : | “N00001” | // (String) |
|  | “ElementAddr” | : | 23 | // (Number) |
|  | “FirmwareVersion” | : | “1.1” | // (String) (Maj.Min) |
|  | “Protocol” | : | DAIKIN216 | // (String) |
|  | “PublishPeriodSec” | : | 300 | // (Number) |
|  | “DeviceUpTimeHrs” | : | “0.54” | // (String) (in hrs) |
|  | “Logging” | : | 1 | // (Number) |
|  | “ResetDevice” | : | 1 | // (Number) |
|  | “ErrorCode” | : | 0 | // (Number) |
| } |  |  |  |  |

Link To Error Code Reference:  Error Codes

Process Flow:

Figure 10.36 Node Debug Info ACK flow

~~~~~~~~~~~ End of Document ~~~~~~~~~~~